#

# Eliminate our own nets from consideration.

#

udp and (not src net 172.21)

and (not src net 172.17.22) and (not src net 172.22)

and (not src net 172.16.26)

and (not src net 172.17.34) and (not src net 172.18.35)

and (not (src port 123 and dst port 123))	# Ignore ntp traffic

and

(

(udp[2:2] < 20)	# destination ports less than 20

or

(udp[0:2] < 20)	# source ports less than 20

or

(dst port 67)       # bootps - Bootstrap Protocol Server

or

(dst port 68)       # bootpc - Bootstrap Protocol Client

or

(dst port 69)       # Trivial file transfer protocol

or

(dst port 88)       # Kerberos

or

(dst port 111)      # sunrpc/portmapper

or

(dst port 135)      # epmap - DCE Endpoint Resolution

or

(dst port 136)      # profile - Profile Naming System

or

#   (dst port 137)	# netbios name service

#   or

(dst port 138)	# netbios datagram service

or

(dst port 139)	# netbios session service

or

(dst port 143)	# IMAP

or

((port 161) or (port 162)) # SNMP

or

(dst port 177)	# xdmcp - X11 logins

or

(dst port 179)	# BGP

or

(dst port 512)	# biff

or

(dst port 513)	# who

or

(dst port 514)	# syslog

or

(dst port 517)	# talk

or

(dst port 518)	# ntalk

or

(dst port 666)	# DOOM

or

(dst port 2049)	# NFS

or

(dst port 3129)	# LDAP

or

(dst port 5631)	# PC anywhere

or

(port 31337) 	# eleet Back Orifice (hacker signature port)

or

(port 18753)

or

(port 20433)

or

(port 34555 or port 35555)	# Back Orifice

or

#

# Crafted packets with impossible UDP lengths

#

(udp[4:2] < 0) or (udp[4:2] > 1500)

)