#

# Eliminate connections coming from ourselves

#

tcp and (not src net 172.21)

and (not src net 172.17.22) and (not src net 172.22)

and (not src net 172.16.26)

and (not src net 172.17.34) and (not src net 172.18.35)

and

(

(not src port 20) 			# FTP data connection

and

(not src port 80)			# Replies to http

and

(tcp[13] & 0x12 = 2)		# Active open (syn is set, ack is not)

and

(

(tcp[2:2] < 20)			# destination ports less than 20

or

(tcp[0:2] < 20)			# source ports less than 20

or

(dst port 22)			# Incoming SSH

or

#

# ftp port    (look for these from undesirable sources or to machines or

#              subnets that shouldnt be receiving ftp)

#

((dst port 21) and

not (

dst host 172.21.107.13   	# host000.goodguys.org

or

dst host 172.21.91.14   	# host001.goodguys.org

or

dst host 172.21.91.15   	# host002.goodguys.org

or

dst host 172.21.49.16   	# host003.goodguys.org

or

dst host 172.21.49.17   	# host004.goodguys.org

or

dst host 172.21.49.18   	# host005.goodguys.org

or

dst host 172.21.49.19   	# host006.goodguys.org

)

)

or

#

# telnet port (look for these from undesirable sources or to machines or

#              subnets that shouldnt be receiving telnets)

#

((dst port 23) and

not (

dst host 172.21.107.13 	# host000.goodguys.org

or

dst host 172.21.91.14 	# host001.goodguys.org

or

dst host 172.21.91.20  	# host007.goodguys.org

or

dst host 172.21.49.16 	# host003.goodguys.org

or

dst host 172.21.49.17 	# host004.goodguys.org

or

dst host 172.21.49.18 	# host005.goodguys.org

or

dst host 172.21.214.21  	# host008.goodguys.org

)

)

or

(dst port 43)				# whois port

or

((dst port 53) and  			# DNS Zone Transfer

not (

dst host 172.21.254.22

or

dst host 172.21.245.23

or

dst host 172.21.254.24

or

dst host 172.22.185.25

or

dst host 172.22.185.26

)

)

or

(dst port 70)				# gopher port

or

(dst port 79)				# finger port

or

(port 87)					# link (hacker signature port)

or

(port 95)					# supdup (hacker signature port)

or

(dst port 109 or dst port 110)		# pop

or

(dst port 111)				# Portmapper

or

#

# NNTP port - (look for these to non-news servers)

#

((dst port 119) and (not dst host 172.21.245.27)

and (not dst host 172.21.245.28)

and (not dst host 172.21.155.29))

or

#

# epmap - DCE Endpoint resolution or Microsoft DCOM

#

(dst port 135)				# MS DCOM NetBIOS

or

(dst port 136)				# profile naming system

or

(dst port 137) 				# netbios name service

or

(dst port 138)   				# netbios datagram service

or

(dst port 139) 				# netbios session service

or

(dst port 143)				# IMAP service

or

(dst port 144)				# NeWS window system

or

(dst port 179)				# Border Gateway Protocol

or

(dst port 421)				# trojaned tcp wrappers

or

#

# Berkeley r-utilities: rexec port 512, rlogin port 512, rshell port 513

#

(dst port 512)

or

(dst port 513)

or

(dst port 514)

or

#

# printer spooler (look for unusual activity)

#

(dst port 515)

or

(dst port 540)				# UUCP

or

(dst port 666)				# Doom and Flash

or

(dst port 749)				# Kerberos Administration

or

(dst port 750)				# Kerberos

or

(dst port 751)				# Kerberos Authentication

or

(dst port 1080)				# SOCKS

or

(dst port 1999)				# cisco ID port

or

(dst port 2000)				# OpenWin

or

(dst port 2049)				# NFS

or

(dst port 2766)				# Listen - the System V Listener

or

(dst port 3128)				# RingZero trojan search

or

(dst port 5232)				# SGI distributed graphics

or

((tcp[2:2] >= 6000) and (tcp[2:2] < 7000))	# X11

or

(port 6665 or port 6666 or port 6667)	# Internet Relay Chat

or

(dst port 7626)				# Glacier Trojan

or

(port 20432)

or

(port 31337)				# eleet (hacker Signature)

or

(port 12345)				# NetBus  ??

or

(port 12346)				# NetBus  ??

or

#

#  Watch for any unknown ports

#

not (

(tcp[2:2] < 20) or

dst port 21 or

dst port 23 or

dst port 22 or

dst port 25 or

dst port 37 or

dst port 43 or

dst port 53 or

dst port 70 or

dst port 79 or

dst port 87 or

dst port 95 or

dst port 109 or

dst port 110 or

dst port 111 or

dst port 113 or

dst port 119 or

dst port 135 or

dst port 136 or

dst port 137 or

dst port 138 or

dst port 139 or

dst port 143 or

dst port 144 or

dst port 443 or

dst port 512 or

dst port 513 or

dst port 514 or

dst port 515 or

dst port 540 or

dst port 563 or

dst port 666 or

dst port 749 or

dst port 750 or

dst port 751 or

dst port 1080 or

dst port 1352 or

dst port 1452 or

dst port 1494 or

dst port 1521 or

dst port 1526 or

dst port 2000 or

dst port 2766 or

dst port 5232 or

((tcp[2:2] >= 6000) and (tcp[2:2] < 7000)) or

dst port 6667 or

dst port 31337 or

dst port 80 or

dst port 8000 or

dst port 8080

)

or

#

#  Impossible Flag filters

#

(tcp[13] & 0x3f = 0) 	# no flags set (null packet)

or

(tcp[13] & 0x11 = 1) 	# FIN set and ACK not set

or

(tcp[13] & 0x03 = 3) 	# SYN set and FIN set

or

(tcp[13] & 0x05 = 5) 	# RST set and FIN set

or

(tcp[13] & 0x06 = 6) 	# SYN set and RST set

or

(tcp[13] & 0x18 = 8) 	# PSH set and ACK not set

or

(tcp[13] & 0x30 = 0x20) 	# URG set and ACK not set

or

(tcp[13] & 0xc0 != 0)	# >= one of the reserved bits of tcp[13] is set

)

)